A ‘box’ model for the past d18O composition of deep waters in the Caspian Sea.

A

B

E

F

V

P

I

A: surface water box with:

input from Volga River (volume V and d18OV),

input from precipitation (volume P and d18OP),

output from evaporation (volume E and d18OE),

exchanges with deep water box (volume F)

B: deep water box with

input from surface water box A (volume F and d18OA)

output from deep water (volume F and d18OB)

The isotope balance equations for box A (d18OA) and for box B (d18OB), assuming a time step of 10 years, are as follows:

d18OB(t+1) = d18OB(t) + d18OA*F – d18OB*F

d18OA(t+1) = d18OA(t) + d18OV * V+ d18OP *P - d18OE*E + d18OB*F – d18OA*F

The fluxes are considered as constant.

The d18OV and d18OP values correspond to the input (I=V+P) of surface water and are calculated from paleoprecipitation data reconstructed from lake Ammersee (von Grafenstein et al., 1999). The isotopic value integrates the Rayleigh distillation evolution (Clark and Fritz, 1997) between lake Ammersee and Caspian Sea.

The isotopic fractionation of d18OE during water evaporation (E) is calculated using the equation by Bottinga and Craig (1969), by Gionfantini (1986) for kinetic isotope fractionation, and  taking into account the value of I calculated previously and the assumptions of a constant evaporation rate and temperature defined for the present day.

Deep-water annual renewal rate (F) represents 7% of the total volume of the Caspian Sea (Peeters et al., 2000).

Calculation of the current isotopic composition of the Caspian Sea water based on the volume and d¹⁸O of input and evaporation and using the equation below, gives a value of -1,55‰ that is consistent with the present-day Caspian water composition.

δ18Ocasp=1000δ18OI1000IE1-hh+dIE1-hh+1

with :

- δ18Ocasp: calculated isotopic composition of the Caspian water taking into account values of volume and d18O of input and evaporation,

- I: total input volume

- δ18OI: isotopic composition of input : d18OI = (d18OV*V+ d18OP*P)/(V+P)

- E: evaporated volume

- h: relative humidity

- εeq: equilibrium isotope fractionation

- εcin: kinetic isotope fractionation

- d: isotopic composition of the evaporated vapor

d=δ18OP1000-εeqh+εeq+εcinh-εeq-εcin

Bottinga, Y. & Craig, H. (1969). Oxygen isotope fractionation between CO₂ and water, and the isotopic composition of marine atmospheric CO₂. Earth and Planetary Science Letters, 5, 285–295. DOI: 10.1016/S0012-821X(68)80054-8.

Clark, I. D. & Fritz, P. (1997). Environmental Isotopes in Hydrogeology. CRC Press/Lewis publisher, ISBN 978-1566702492, 342p.

Gonfiantini, R., 1986. Environmental isotopes in Lake studies, in P. Fritz et J.-Ch. Fontes (dir), Handbook of environmental isotope geochemistry, vol 2, p 113-168, ISSN 0167-949X.